- SECTION 1: IDENTIFICATION

- 1.1.	Product Identifier

Product Form: Mixture

Product Name: Household Ammonia, 4.12% Ammonium Hydroxide

Synonyms: Household Ammonia, Aqua Ammonia

- 1.2.	Intended Use of the Product

Use of the substance/mixture: Hard surface cleaner, deodorizer

- 1.3.	Name, Address, and Telephone of the Responsible Party

| - Company - OnLine Packaging, Inc. 4311 Liberty Lane Plover, WI 54467 T: (715)344-4861 www.onlinepackaging.org |  |
| --- | --- |

- 1.4.	Emergency Telephone Number

| Emergency Number | : | (715) 344-4861 Call doctor/hospital emergency room or the Local Poison Control Center. Have the product container or label with you when calling a Poison Control Center or doctor, or when going for treatment. |
| --- | --- | --- |

SECTION 2: HAZARDS IDENTIFICATION

- 2.1.	Classification of the Substance or Mixture

- Classification (GHS-US)

| Skin Irrit. 2 | H315 |
| --- | --- |
| Eye Irrit. 2A | H319 |
| Aquatic Acute 3 | H402 |

Full text of H-phrases: see section 16

- 2.2.	Label Elements

- GHS-US Labeling

| Hazard Pictograms (GHS-US) | : | GHS07 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Signal Word (GHS-US) | : | Warning |  |  |  |  |  |
| Hazard Statements (GHS-US) | : | H315 - Causes skin irritation. H319 - Causes serious eye irritation. H402 - Harmful to aquatic life. |  |  |  |  |  |
| Precautionary Statements (GHS-US) | : | P264 - Wash hands, forearms, and other exposed areas thoroughly after handling. P273 - Avoid release to the environment. P280 - Wear protective gloves, protective clothing, and eye protection. P302+P352 - If on skin: Wash with plenty of water. P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P321 - Specific treatment (see section 4 on this SDS). P332+P313 - If skin irritation occurs: Get medical advice/attention. P337+P313 - If eye irritation persists: Get medical advice/attention. P362 - Take off contaminated clothing and wash before reuse. P501 - Dispose of contents/container in accordance with local, regional, national, provincial, territorial and international regulations. |  |  |  |  |  |

- 2.3.	Other Hazards

Exposure may aggravate those with pre-existing eye, skin, or respiratory conditions.

- 2.4.	Unknown Acute Toxicity (GHS-US)

No data available

SECTION 3: COMPOSITION/INFORMATION ON INGREDIENTS

- 3.1.	Substances

Not applicable

- 3.2.	Mixture*

| Name | Product Identifier | % (w/w) | Classification (GHS-US) |
| --- | --- | --- | --- |
| Ammonia | (CAS No) 7664-41-7 | 2 | Flam. Gas 2, H221 Liquefied gas, H280 Acute Tox. 3 (Inhalation:gas), H331 Skin Corr. 1B, H314 Eye Dam. 1, H318 STOT SE 3, H335 Aquatic Acute 1, H400 Aquatic Chronic 2, H411 |
| Water | (CAS No) 7732-18-5 | 98 | Not classified |

*4.12% Ammonium Hydroxide (CAS No) 1336-21-6 is formed by blending Anhydrous Ammonia with water in the proportions listed.

Full text of H-phrases: see section 16

- SECTION 4: FIRST AID MEASURES

- 4.1.	Description of First Aid Measures

First-aid Measures General: Never give anything by mouth to an unconscious person. If you feel unwell, seek medical advice (show the label if possible).

First-aid Measures After Inhalation: Remove to fresh air and keep at rest in a position comfortable for breathing. Obtain medical attention if breathing difficulty persists.

First-aid Measures After Skin Contact: Rinse immediately with plenty of water for at least 15 minutes. Obtain medical attention if irritation develops or persists.

First-aid Measures After Eye Contact: Rinse cautiously with water for at least 15 minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Obtain medical attention.

First-aid Measures After Ingestion: Do NOT induce vomiting. Rinse mouth. Immediately call a POISON CENTER or doctor.

- 4.2.	Most important symptoms and effects, both acute and delayed

Symptoms/Injuries: Causes serious eye irritation. Causes skin irritation. Ammonia is a severe irritant of the eyes, respiratory tract and skin. It may cause burning and tearing of the eyes, runny nose and chest pain. Exposure of the eyes in high concentrations may produce temporary blindness (cataracts and glaucoma) and severe eye damage. Exposure of the skin may cause burning and blistering of the skin.

Symptoms/Injuries After Inhalation: May cause respiratory irritation.

Symptoms/Injuries After Skin Contact: Causes skin irritation. Redness, pain, swelling, itching, burning, dryness, and dermatitis.

Symptoms/Injuries After Eye Contact: Causes serious eye irritation. Redness, pain, swelling, itching, burning, tearing, and blurred vision.

Symptoms/Injuries After Ingestion: Ingestion is likely to be harmful or have adverse effects.

Chronic Symptoms: None expected under normal conditions of use.

- 4.3.	Indication of Any Immediate Medical Attention and Special Treatment Needed

If you feel unwell, seek medical advice (show the label where possible).

- SECTION 5: FIRE-FIGHTING  MEASURES

- 5.1.	Extinguishing Media

Suitable Extinguishing Media: Use extinguishing media appropriate for surrounding fire.

Unsuitable Extinguishing Media: Do not use a heavy water stream. Use of heavy stream of water may spread fire. Application of water stream to hot product may cause frothing and increase fire intensity.

- 5.2.	Special Hazards Arising From the Substance or Mixture

Fire Hazard: Not considered flammable but may burn at high temperatures.

Explosion Hazard: Product is not explosive.

Reactivity: Hazardous reactions will not occur under normal conditions.

- 5.3.	Advice for Firefighters

Precautionary Measures Fire: Exercise caution when fighting any chemical fire.

Firefighting Instructions: Use water spray or fog for cooling exposed containers.

Protection During Firefighting: Do not enter fire area without proper protective equipment, including respiratory protection.

Other Information: Thermal decomposition may produce corrosive vapors of ammonia and toxic oxides of nitrogen.

- SECTION 6: ACCIDENTAL RELEASE MEASURES

- 6.1.	Personal Precautions, Protective Equipment and Emergency Procedures

General Measures: Avoid all contact with skin, eyes, or clothing. Avoid breathing (vapor, mist, spray).

- 6.1.1.	For Non-emergency Personnel

Protective Equipment: Use appropriate personal protection equipment (PPE).

Emergency Procedures: Evacuate unnecessary personnel.

6.1.2.	For Emergency Responders

Protective Equipment: Equip cleanup crew with proper protection.

Emergency Procedures: Stop leak if safe to do so. Eliminate ignition sources. Ventilate area.

- 6.2.	Environmental Precautions

Prevent entry to sewers and public waters. Notify authorities if liquid enters sewers or public waters.

- 6.3.	Methods and Material for Containment and Cleaning Up

For Containment: Contain any spills with dikes or absorbents to prevent migration and entry into sewers or streams.

Methods for Cleaning Up: Clean up spills immediately and dispose of waste safely. Spills should be contained with mechanical barriers. Transfer spilled material to a suitable container for disposal. Contact competent authorities after a spill.

- 6.4.	Reference to Other Sections

See Heading 8. Exposure controls and personal protection. For further information refer to section 13.

- SECTION 7: HANDLING AND STORAGE

- 7.1.	Precautions for Safe Handling

Hygiene Measures: Handle in accordance with good industrial hygiene and safety procedures. Wash hands and other exposed areas with mild soap and water before eating, drinking or smoking and when leaving work.

- 7.2.	Conditions for Safe Storage, Including Any Incompatibilities

Technical Measures: Comply with applicable regulations.

Storage Conditions: Store in a dry, cool and well-ventilated place. Keep container closed when not in use. Keep/Store away from direct sunlight, extremely high or low temperatures and incompatible materials.

Incompatible Products: Strong acids. Strong bases. Strong oxidizers. Nonferrous metals, alkali metal, bleaches, and toilet bowl cleaners. . Acids, Halogens, salts of silver and zinc. Corrosive to copper and galvanized surfaces. Reacts exothermically with acids. Hydrochloric Acid. Brass, zinc, alloys containing copper. Sodium hypochlorite/hypochlorite’s.

7.3.	Specific End Use(s)

Hard surface cleaner, deodorizer

- SECTION 8: EXPOSURE CONTROLS/PERSONAL PROTECTION

- 8.1.	Control Parameters

For substances listed in section 3 that are not listed here, there are no established exposure limits from the manufacturer, supplier, importer, or the appropriate advisory agency including: ACGIH (TLV), NIOSH (REL), or OSHA (PEL).

| Ammonia (7664-41-7) |  |  |
| --- | --- | --- |
| USA ACGIH | ACGIH TWA (ppm) | 25 ppm |
| USA ACGIH | ACGIH STEL (ppm) | 35 ppm |
| USA NIOSH | NIOSH REL (TWA) (mg/m³) | 18 mg/m³ |
| USA NIOSH | NIOSH REL (TWA) (ppm) | 25 ppm |
| USA NIOSH | NIOSH REL (STEL) (mg/m³) | 27 mg/m³ |
| USA NIOSH | NIOSH REL (STEL) (ppm) | 35 ppm |
| USA IDLH | US IDLH (ppm) | 300 ppm |
| USA OSHA | OSHA PEL (TWA) (mg/m³) | 35 mg/m³ |
| USA OSHA | OSHA PEL (TWA) (ppm) | 50 ppm |

8.2.	Exposure Controls

| Appropriate Engineering Controls | : | Ensure adequate ventilation, especially in confined areas. Emergency eye wash fountains and safety showers should be available in the immediate vicinity of any potential exposure. Ensure all national/local regulations are observed. |
| --- | --- | --- |
| Personal Protective Equipment | : | Protective goggles. Gloves. Protective clothing. |
| Materials for Protective Clothing | : | Chemically resistant materials and fabrics. |
| Hand Protection | : | Wear chemically resistant protective gloves. |
| Eye Protection | : | Chemical safety goggles. |
| Skin and Body Protection | : | Wear suitable protective clothing. |
| Respiratory Protection | : | Use a NIOSH-approved respirator or self-contained breathing apparatus whenever exposure may exceed established Occupational Exposure Limits. |
| Environmental Exposure Controls | : | Do not allow the product to be released into the environment. |
| Consumer Exposure Controls | : | Do not eat, drink or smoke during use. |

- SECTION 9: PHYSICAL AND CHEMICAL PROPERTIES

- 9.1.	Information on Basic Physical and Chemical Properties

| Physical State | : | Liquid |
| --- | --- | --- |
| Appearance | : | Clear, Cloudy/White or Yellow Liquid |
| Odor | : | Ammonia |
| Odor Threshold | : | No data available |
| pH | : | 11.2 - 11.8 |
| Evaporation Rate | : | No data available |
| Melting Point | : | No data available |
| Freezing Point | : | No data available |
| Boiling Point | : | No data available |
| Flash Point | : | No data available |
| Auto-ignition Temperature | : | No data available |
| Decomposition Temperature | : | No data available |
| Flammability (solid, gas) | : | No data available |
| Vapor Pressure | : | No data available |
| Relative Vapor Density at 20 °C | : | No data available |
| Relative Density | : | No data available |
| Specific Gravity | : | 0.9913 |
| Solubility | : | Water: Complete |
| Partition Coefficient: N-Octanol/Water | : | No data available |
| Viscosity | : | No data available |

9.2.	Other Information  No additional information available

- SECTION 10: STABILITY AND REACTIVITY

- 10.1.      Reactivity:    Hazardous reactions will not occur under normal conditions. May react hazardously with incompatibles.

- 10.2.      Chemical Stability:    Stable under recommended handling and storage conditions (see section 7).

- 10.3.      Possibility of Hazardous Reactions:    Hazardous polymerization will not occur.

- 10.4.      Conditions to Avoid:    Direct sunlight. Extremely high or low temperatures. Ignition sources. Incompatible materials.

- 10.5.      Incompatible Materials:    Strong acids. Strong bases. Strong oxidizers. Nonferrous metals, alkali metal, bleaches, and toilet bowl cleaners. Acids, Halogens, salts of silver and zinc. Corrosive to copper and galvanized surfaces. Reacts exothermically with acids.  Hydrochloric Acid. Brass, zinc, alloys containing copper. Sodium hypochlorite/hypochlorite’s.

10.6.      Hazardous Decomposition Products:    Reacts exothermically with acids.  Nitric oxide (NOx) and Ammonia (NH3) fumes. Thermal decomposition may produce corrosive vapors of ammonia and toxic oxides of nitrogen.

- SECTION 11: TOXICOLOGICAL INFORMATION

- 11.1.	Information On Toxicological Effects

Acute Toxicity: Not classified

| Ammonia (7664-41-7) |  |
| --- | --- |
| LC50 Inhalation Rat | 5.1 mg/l (Exposure time: 1 h) |
| LC50 Inhalation Rat | 2000 ppm/4h (Exposure time: 4 h) |

| Ammonium hydroxide (1336-21-6) |  |
| --- | --- |
| LD50 Oral Rat | 350 mg/kg |

Skin Corrosion/Irritation: Causes skin irritation.

pH: 11.2 - 11.8

Serious Eye Damage/Irritation: Causes serious eye irritation.

pH: 11.2 - 11.8

Respiratory or Skin Sensitization: Not classified

Germ Cell Mutagenicity: Not classified

Carcinogenicity: Not classified

Reproductive Toxicity: Not classified

Specific Target Organ Toxicity (Single Exposure): Not classified

Specific Target Organ Toxicity (Repeated Exposure): Not classified

Aspiration Hazard: Not classified

Symptoms/Injuries After Inhalation: May cause respiratory irritation.

Symptoms/Injuries After Skin Contact: Causes skin irritation. Redness, pain, swelling, itching, burning, dryness, and dermatitis.

Symptoms/Injuries After Eye Contact: Causes serious eye irritation. Redness, pain, swelling, itching, burning, tearing, and blurred vision.

Symptoms/Injuries After Ingestion: Ingestion is likely to be harmful or have adverse effects.

Chronic Symptoms: None expected under normal conditions of use.

- SECTION 12: ECOLOGICAL INFORMATION

- 12.1.	Toxicity

| Ecology - General | : | Harmful to aquatic life. |
| --- | --- | --- |

| Ammonia (7664-41-7) |  |
| --- | --- |
| LC50 Fish 1 | 0.44 mg/l (Exposure time: 96 h - Species: Cyprinus carpio) |
| EC50 Daphnia 1 | 25.4 mg/l (Exposure time: 48 h - Species: Daphnia magna) |
| LC 50 Fish 2 | 0.26 - 4.6 mg/l (Exposure time: 96 h - Species: Lepomis macrochirus) |

| Ammonium hydroxide (1336-21-6) |  |
| --- | --- |
| LC50 Fish 1 | 8.2 mg/l (Exposure time: 96 h - Species: Pimephales promelas) |
| EC50 Daphnia 1 | 0.66 mg/l (Exposure time: 48 h - Species: water flea) |
| EC50 Daphnia 2 | 0.66 mg/l (Exposure time: 48 h - Species: Daphnia pulex) |

- 12.2.	Persistence and Degradability  No additional information available

- 12.3.	Bioaccumulative Potential

| Ammonia (7664-41-7) |  |
| --- | --- |
| Log Pow | -1.14 (at 25 °C) |

- 12.4.	Mobility in Soil  No additional information available

12.5.	Other Adverse Effects

| Other Information | : | Avoid release to the environment. |
| --- | --- | --- |

- SECTION 13: DISPOSAL CONSIDERATIONS

- 13.1.	Waste treatment methods

Waste Disposal Recommendations: Dispose of waste material in accordance with all local, regional, national, and international regulations.

- SECTION 14: TRANSPORT INFORMATION

14.1.     In Accordance with DOT       Not regulated for transport

SECTION 15: REGULATORY INFORMATION

- 15.1      US Federal Regulations

| Ammonia (7664-41-7) |  |
| --- | --- |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory Listed on the United States SARA Section 302 Listed on United States SARA Section 313 |  |
| SARA Section 302 Threshold Planning Quantity (TPQ) | 500 |
| SARA Section 311/312 Hazard Classes | Fire hazard Immediate (acute) health hazard Sudden release of pressure hazard |
| SARA Section 313 - Emission Reporting | 1.0 % (includes anhydrous Ammonia and aqueous Ammonia from water dissociable Ammonium salts and other sources, 10% of total aqueous Ammonia is reportable under this listing) |

Water (7732-18-5)

Listed on the United States TSCA (Toxic Substances Control Act) inventory

| Ammonium hydroxide (1336-21-6) |  |
| --- | --- |
| Listed on the United States TSCA (Toxic Substances Control Act) inventory |  |
| SARA Section 311/312 Hazard Classes | Immediate (acute) health hazard |

- 15.2      US State Regulations

Ammonia (7664-41-7)

U.S. - Massachusetts - Right To Know List U.S. - New Jersey - Right to Know Hazardous Substance List U.S. - Pennsylvania - RTK (Right to Know) - Environmental Hazard List U.S. - Pennsylvania - RTK (Right to Know) List

Ammonium hydroxide (1336-21-6)

U.S. - Massachusetts - Right To Know List U.S. - New Jersey - Right to Know Hazardous Substance List U.S. - Pennsylvania - RTK (Right to Know) - Environmental Hazard List U.S. - Pennsylvania - RTK (Right to Know) List

SECTION 16: OTHER INFORMATION, INCLUDING DATE OF PREPARATION OR LAST REVISION

| Revision Date |  |  | : | 04/08/2015 |
| --- | --- | --- | --- | --- |
| Other Information |  |  | : | This document has been prepared in accordance with the SDS requirements of the OSHA Hazard Communication Standard 29 CFR 1910.1200. |
| GHS Full Text Phrases: |  |  |  |  |
| ------ | Acute Tox. 3 (Inhalation:gas) | Acute toxicity (inhalation:gas) Category 3 |  |  |
| ------ | Aquatic Acute 1 | Hazardous to the aquatic environment - Acute Hazard Category 1 |  |  |
| ------ | Aquatic Acute 3 | Hazardous to the aquatic environment - Acute Hazard Category 3 |  |  |
| ------ | Aquatic Chronic 2 | Hazardous to the aquatic environment - Chronic Hazard Category 2 |  |  |
| ------ | Eye Dam. 1 | Serious eye damage/eye irritation Category 1 |  |  |
| ------ | Eye Irrit. 2A | Serious eye damage/eye irritation Category 2A |  |  |
| ------ | Flam. Gas 2 | Flammable gases Category 2 |  |  |
| ------ | Liquefied gas | Gases under pressure Liquefied gas |  |  |
| ------ | Skin Corr. 1B | Skin corrosion/irritation Category 1B |  |  |
| ------ | Skin Irrit. 2 | Skin corrosion/irritation Category 2 |  |  |
| ------ | STOT SE 3 | Specific target organ toxicity (single exposure) Category 3 |  |  |
| ------ | H221 | Flammable gas |  |  |
| ------ | H280 | Contains gas under pressure; may explode if heated |  |  |
| ------ | H314 | Causes severe skin burns and eye damage |  |  |
| ------ | H315 | Causes skin irritation |  |  |
| ------ | H318 | Causes serious eye damage |  |  |
| ------ | H319 | Causes serious eye irritation |  |  |
| ------ | H331 | Toxic if inhaled |  |  |
| ------ | H335 | May cause respiratory irritation |  |  |
| ------ | H400 | Very toxic to aquatic life |  |  |
| ------ | H402 | Harmful to aquatic life |  |  |
| ------ | H411 | Toxic to aquatic life with long lasting effects |  |  |

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.

SDS US (GHS HazCom)